# Automation & Operational Technology

# WAN Link – Work Package

### Document Control

| Version No. for Final Release | 1.0 |
| --- | --- |
| Issue Date | 01-01-2026 |
| Status: Draft or Final | Draft |
| Document Owner | UID |
| Audience | SbM |
| Approval for Final Release | Draft |

### Project Overview / Warehouse Details

Project Name:

FBM ID:

Site address:

SITE ID:

Dates:

xx/xx/2026 – Access to Site

xx/xx/2026 – Internet Services Need by Date

xx/xx/2026 – Go live.

## 1. SCOPE OF WORK

The vendor shall design, deliver, implement, and support a resilient SD-WAN (Internet) connectivity solution for Maersk warehouse operations. The solution must ensure secure, redundant, high-performance connectivity enabling WMS, printing, RF devices, cloud workloads, and remote support.

### 2 Functional requirements

- WAN must provide high level of service availability

- WAN services must have redundancy for down time reduction.

- Meet site workload needs with committed bandwidth, low packet loss, and latency suitable for WMS, printing, and RF devices.

- Enable continuous monitoring and alerting for link status, utilization, and SLA breaches

- 24×7 support, including NOC monitoring

- Segregate business-critical traffic from guest/non-critical traffic to protect performance.

### 3 Technical requirements

- Carrier Diversity: Single / Dual

- Quantity of Internet Links: 1 / 2/ 3

- Diversity: Router / Full Path / Partial Path

- Public IPs:  2 / 3 / More

- Target Site Baselines:

- ≥500 Mbps (bandwidth) total Internet capacity.

- Latency <100 ms (general), <300 ms for cloud access

- jitter and loss within typical enterprise thresholds.

- 99.99% or higher service availability

- Terminate WAN on edge firewalls/routers

- Enable stateful inspection and QoS/traffic shaping for critical applications (e.g., WMS, label printing, voice/video if applicable).

- Provide link telemetry (SNMP/NetFlow/API) into central monitoring; keep change/config logs and monthly utilization/SLA reports.

- Automated failover within <300ms (SD-WAN) or <1s (BGP)

- Circuit should be delivered in: MDF or Electrical Room – Appendix X

## 4. Deliverables

### 4.1 MS1.3 - PROV OF DESIGN

Procurement, Design and delivery WAN Link defined in Section 1 to 3

Physical installation of internet services and routers in designated locations

Installation of peripherals, accessories as required

### 4.2 MS2 - FAT

Configuration of Router and Services as per technical requirements

### 4.3 MS3 - SAT

Validation that delivered services and router meet Section 3 Requirements.

All equipment must be validated and ready for operational use prior to Go‑Live.

### 4.4 MS4 - UAT

Asset inventory including Device type, Model, Serial number.

Mapping of devices

Handover of equipment to Network team

Provision of support and escalation contacts

### 4.5 MS5 - HYPERCARE

Confirm NOC Monitoring and SLA reports

Provision of support and escalation contacts

## 5. Appendix

### Appendix A: MDF / Electrical Room location / MPOE Room

Detailed information of conduits, paths and locations required for the delivery of the Internet services to a warehouse.