# Automation & Operational Technology

# Perimeter Firewall – Work Package

### Document Control

| Version No. for Final Release | 1.0 |
| --- | --- |
| Issue Date | 01-01-2026 |
| Status: Draft or Final | Draft |
| Document Owner | UID |
| Audience | SbM |
| Approval for Final Release | Draft |

### Project Overview / Warehouse Details

Project Name:

FBM ID:

Site address:

SITE ID:

Dates:

xx/xx/2026 – Access to Site

xx/xx/2026 – Perimeter Services Need by Date

xx/xx/2026 – Go live.

## 1. SCOPE OF WORK

The vendor shall design, deliver, configure, and support a secure and highly‑available perimeter firewall and router infrastructure for Maersk warehouse operations. The solution must protect IT and OT environments, ensure uninterrupted connectivity to critical systems, and enforce enterprise‑grade security standards.

### 2 Functional requirements

- Protect perimeter and IT / OT work devices with policy-based access control; only allow required services.

- Maintain secure connectivity to cloud/vendor systems (e.g., AWS Outposts/Scout, AFTX, SOTI) while blocking unauthorized traffic.

- Provide high availability to avoid single points of failure during maintenance or outages.

- Support site operations with simple rules for printing/scanning and role-based administration for IT/OT teams.

- Produce auditable logs and alerts for security events, VPN status, and policy changes.

- Enable Secure Access for Vendors to OT Machines and/or PLCs.

- Provide resilient internet connectivity for WMS, remote support, and cloud services; maintain operations during ISP outages via automatic failover

- NOC Monitoring and 99.9% SLA uptime

### 3 Technical requirements

- High Availability Capabilities (Active-Active or Active/Passive; Dual Power Supplies)

- Core NGFW Features (deep packet inspection, advanced IPS/IDS, TLS 1.3 inspection, botnet detection, Geo-IP filtering, Anti-malware and sandboxing).

- Industrial protocol coverage (Modbus, Profinet, EtherNet/IP, OPC-UA, CIPs). - If Applicable

- Segmentation Capabilities (including Zero Trust)

- Purdue Model enforcement (Clear separation of Lvl1 to Lvl4 – ISA95)

- Standard Routing & Switching (BGP, OSPF, Static Routing, ECMP, VRFs, etc.)

- VPON / Remote Access (IPsec, SSL VPN, MFA, VRF/Zone for vendor access isolation)

- SD-WAN Integration Capabilities

- Logging, Monitoring and SIEM Integration capabilities.

- Centralized Management (administration and policies)

- Throughput for Bandwidth: XXXXXX

- VPN Requirements – Appendix X

- Segmentation Requirements – Appendix X1

- Special requirements:

  - XXXXX

  - XXXXXX

  - XXXXXXX

## 4. Deliverables

### 4.1 MS1.3 – Provision of Design & Procurement

Procurement, design and delivery of all equipment as defined in Sections 1–3

Physical installation of equipment in designated locations

Installation of accessories and peripherals as required

### 4.2 MS2 – FAT (Factory Acceptance Testing)

Factory acceptance testing and verification of all hardware prior to site delivery

Pre-staging and configuration of equipment as per technical requirements

Confirmation of full Bill of Materials receipt and manufacturer warranty coverage

### 4.3 MS3 – SAT (Site Acceptance Testing & Commissioning)

Physical installation and commissioning of all equipment on site

Connectivity and integration testing (network, power, and application layer)

Validation that all delivered equipment meets Section 3 requirements

All equipment must be validated and ready for operational use prior to Go-Live

### 4.4 MS4 – UAT (User Acceptance Testing)

Operations user acceptance testing

Asset inventory including: Device type, Model, Serial number

Mapping of devices to: Users, Stations, Rooms or operational areas

Handover of equipment to site management and operations team

Provision of support and escalation contacts

### 4.5 MS5 – Hypercare & Project Closure

Hypercare support during initial operational period

Confirm NOC monitoring and SLA reports (where applicable)

Validate all documentation required for handover to support is complete

Final project acceptance sign-off and closure

## 5. Appendix

### Appendix X: MDF / Electrical Room location / MPOE Room

Detailed information of conduits, paths and locations required for the delivery of the Internet services to a warehouse.