# Automation & Operational Technology

# Active Network Work Package

### Document Control

| Version No. for Final Release |  |
| --- | --- |
| Issue Date |  |
| Status: Draft or Final |  |
| Document Owner |  |
| Audience |  |
| Approval for Final Release |  |

### Project Overview / Warehouse Details

Project Name:

FBM ID:

Site address:

SITE ID:

Dates:

MS1.1 – Provision of Supplier - xx/xx/2026

MS1.2 – Provision of Design - xx/xx/2026

MS2 – Factory Acceptance / Lab Testing / Simulation - xx/xx/2026

MS3 – Unit Testing - xx/xx/2026

MS4.1 – Site Integration - xx/xx/2026

MS4.2 – UAT (User Acceptance Testing) - xx/xx/2026

MS5 – Hypercare - xx/xx/2026

MS6 – Business as Usual Support  - xx/xx/2026

## 1. SCOPE OF WORK

Objective: To build a resilient, high-availability network solution that integrates IT, Operational Technology (OT), and Automation services, can seamlessly manage shop-floor hardware.

## 2. FUNCTIONAL REQUIREMENTS

To proceed with procurement and installation, the Network Team must provide:

- LAN Bill of Materials (BoM): Comprehensive list of Firewalls, Stackable Core Switches, Access Switches (PoE+), and SFP+ transceivers.

- WLAN Bill of Materials (BoM): Access Point (AP) count and specific mounting brackets (Beam, Wall, or Ceiling mounts).

- Predictive WLAN Heatmap: RF simulation showing signal strength (dBm) and Signal-to-Noise Ratio (SNR) for the entire facility.

- Logical Network Diagram: Visualizing the VLAN schema and Inter-VLAN routing policies.

## 3. TECHNICAL REQUIREMENTS

## 3.1. High Availability & Redundancy

- ISP Redundancy: Design for two (2) independent Internet Service Providers with diverse physical entry points to the MDF.

- Redundant Core: Firewalls must be deployed in a High Availability (HA) Cluster. Core switches must be Stackable to provide hardware redundancy with single-IP management.

- IDF Strategy: Strategically locate IDFs (Intermediate Distribution Frames) to ensure all Access Points and AOT data drops comply with the 90-meter (295 ft) copper limit.

## 3.2. VLAN Segmentation & Logical Requirements

The following VLANs are required as a baseline for security and traffic prioritization:

- User Wired: Corporate office/workstation data.

- User WLAN: Corporate Wi-Fi for laptops and mobile staff.

- Printers: Dedicated segment for paper and label printers.

- RDT (RF Scanners): High-priority QoS segment for handheld devices.

- OT / Automation: PLCs, Conveyors, Put-to-Light, and Cubiscan units.

- Server / iPC: Local hosts or industrial PCs (if applicable).

- Management: Out-of-band management for all switches, UPS, and APs.

### 3.3. Inter-VLAN Communication (if applicable)

The design must enable controlled routing to support Maersk Control System (MCS) operations:

- Functional Integration: Enable bidirectional routing between the MCS/Server VLAN and the OT/Automation VLAN.

- Use Case: This is required for MCS to communicate directly with PLCs and RFID Readers for real-time sorter and conveyor management.

- Security: Apply Access Control Lists (ACLs) to ensure only authorized MCS protocols can cross-segment.

## 3.4 OT Net & Machine Net:

To ensure proper integration between IT infrastructure and Operational Technology (OT) systems, all required network ports and communication protocols must be explicitly considered in the network design, firewall configuration, and security policies.

#### 3.4.1 Standard OT Communication Ports

The following ports must be allowed and properly routed between relevant VLANs (e.g., OT/Automation VLAN and MCS/Server VLAN), based on equipment requirements:

| Device / System | Required Ports | Protocol / Notes |
| --- | --- | --- |
| PLC | TCP 102 | Siemens S7 / Industrial control |
| RFID Readers | TCP 80, 443 | HTTP/HTTPS communication |
| Cubiscan / MyCubi | TCP 80, 443 | API / Web services |
| AIOI PTW / PTL | TCP 5003 | Vendor-specific control protocol |
| MCS (Control System) | TCP 80, 443 | Web services / orchestration |

#### 3.4.2 Network and Security Considerations

- All listed ports must be explicitly enabled via firewall rules and ACLs, following a least privilege model

- Communication must be restricted only between required source and destination VLANs

- Deep Packet Inspection (DPI) or protocol-aware filtering should be considered where applicable

- Any additional vendor-specific ports must be documented and approved prior to implementation

#### 3.4.3 Industrial Network Requirements

If an industrial network is required for PLC communication or real-time automation systems:

- Industrial-grade switches must be deployed, capable of supporting PROFINET, Modbus TCP, or equivalent industrial Ethernet protocols

- Switches must support:

  - Low latency and deterministic traffic handling

  - IGMP Snooping / Multicast control (critical for PROFINET)

  - QoS prioritization for OT traffic

- Network design must avoid oversubscription and ensure predictable performance for control traffic

#### 3.4.4 Design Constraints

- OT traffic must be logically segmented (dedicated VLAN) and isolated from corporate traffic

- Inter-VLAN routing must be tightly controlled and only enabled for required services (e.g., MCS ↔ PLC communication)

- Latency-sensitive traffic (PLC, PTL systems) must be prioritized over standard data traffic

### 3.5 WLAN PERFORMANCE FOR MOBILITY

- Standard: Wi-Fi 6 (802.11ax) optimized for RDT (Radio Data Terminals) and Laptops.

- Roaming: Design for sub-second L2/L3 roaming to prevent session timeouts while scanners move between high-density racking aisles.

- Mounting Heights: The Heatmap must specify AP mounting heights that clear industrial obstructions like overhead conveyors, HVAC ducts, and fire sprinklerers.

### 3.6 TECHNICAL CONSTRAINTS FOR THE DESIGN

- Cabling Standard: Design must align with Cat 6A F/UTP shielded cabling to mitigate industrial Electromagnetic Interference (EMI).

- Fiber Backbone: MDF-to-IDF interconnections must utilize 6-core OM4 Multimode Fiber.

- PoE Budget: Access switch selection must account for full PoE+ load for APs and OT devices, plus a 20% growth margin.

- Power: UPS in each IDF must provide 15 minutes of backup time for all active equipment.

## 4. Deliverables

Planning

MS1.1 – Provision of Supplier – delivery date

- Full Bill of Materials (BoM) submitted and validated, including manufacturer, model, part numbers, port/component specifications, and compliance certifications

- Approved Purchase Orders confirmed for all equipment and professional services

MS1.2 – Provision of Design - delivery date

- Design documentation issued: network topology, connectivity diagrams, rack elevation diagrams, floor plan layouts, IP schema, VLAN and segmentation design, and cable routing plans

- Procurement and delivery of all equipment as defined in Sections 1–3 initiated and tracked against BoM

Execute

MS2 – Factory Acceptance / Lab Testing / Simulation - delivery date

- Full BoM receipt confirmed; manufacturer warranty coverage verified for all equipment and components

- Pre-staging and configuration completed as per Section 3 technical requirements

- Factory acceptance testing executed and signed off prior to site delivery

MS3 – Unit Testing - delivery date

- Physical installation and commissioning of all equipment on site, including accessories, peripherals, mounting hardware, and cabling

- As-built drawings and installation photos produced, reflecting final installed routes, placements, termination points, and labelling

- Certified test reports produced for all applicable runs and systems (e.g. Fluke cable certification, UPS runtime, RF coverage, VLAN and uplink validation)

- Compliance documentation confirmed against applicable standards (ANSI/TIA-568, ISO/IEC 11801, fire safety, and any OT-specific industrial standards)

MS4.1 – Site Integration - delivery date

- Connectivity and integration testing completed across network, power, and application layers

- Validation evidence confirms all delivered equipment meets Section 3 requirements; outstanding items logged in a punch list with agreed resolution dates

MS4.2 – UAT (User Acceptance Testing) - delivery date

- Operations UAT completed and signed off by site management

- Asset inventory compiled: device type, model, serial number, firmware version, and registered in snow CMDB

- Devices mapped to assigned users, workstations, rooms, or operational areas

- RF Guns/RDT devices registered in SOTI as a mandatory task

- All documentation delivered in soft copy formats

- Formal handover of equipment to site management, including credentials and escalation contacts

Go Live

MS5 – Hypercare - delivery date

- Hypercare support active during agreed initial operational period; incidents logged and resolved

- NOC monitoring confirmed active and initial SLA report issued (where applicable)

- All handover documentation validated as complete: as-built drawings, configuration backups, test reports, compliance records, asset register, and warranty certificates

MS6 – Business as Usual Support - delivery date

- Final project acceptance signed off; project formally closed

- Support transitioned to BAU team with full handover pack, escalation matrix, and SLA documentation in place

- All outstanding punch list items resolved or formally accepted and transitioned to BAU tracking

## 5. Appendix

### Appendix A: Automation Layout

Detailed list of all IT hardware, including:

End‑user devices

Warehouse operational equipment

Printers and meeting room solutions

Accessories

### Appendix B: Quantity Overview

Summary of quantities for:

Administrative users

Pack, Inbound, and Return stations

Printers and meeting rooms

### Appendix C: Office & Warehouse Layouts

Office floor layouts

Warehouse workstation locations

Meeting room locations

### Appendix D: XXXX Specific Requirements

Zxxxxxyyyy